Airbus LiDAR Obstacle Detection – Technical Summary

Problem

The Airbus challenge requires detecting infrastructure obstacles from airborne LiDAR point clouds and outputting 3D bounding boxes for four classes:

- Antenna

- Cable

- Electric Pole

- Wind Turbine

The dataset provides 10 training scenes (~100 frames each) stored as HDF5 LiDAR scans. Training data contains point-wise semantic labels encoded as RGB values, while the evaluation dataset removes RGB information.

Evaluation metrics include:

- mAP @ IoU = 0.5

- Mean IoU

- Robustness across point densities (100%, 75%, 50%, 25%)

- Model efficiency (parameter count)

- Runtime stability

Key Challenges

1. No Ground Truth 3D Boxes: Only point-wise RGB labels are provided. Bounding boxes must be reconstructed.

2. Small Dataset: Only 10 scenes are available for training.

3. Thin Structures: Cable detection is difficult due to sparse points and fragmentation.

4. Density Variation: Evaluation includes heavy point subsampling.

5. RGB Removed at Evaluation: Final detection must rely only on geometry.

Method Overview

We implemented a hybrid geometry + lightweight learning pipeline. The key idea is to exploit strong geometric priors rather than relying solely on deep learning.

Pipeline:

LiDAR HDF5 → Frame extraction → Spherical to Cartesian conversion → RGB to point labels (training) →

Two strategies:

(1) Geometry-first heuristic detector

(2) Lightweight point segmentation model

→ Class-aware clustering → Oriented bounding boxes.

Geometry-Based Detector (Baseline)

The main detection pipeline relies on geometry-aware clustering:

1. Voxel downsampling

2. Class-wise point filtering

3. DBSCAN clustering

4. PCA for orientation

5. Oriented bounding box fitting

Cable Detection

Cables are thin linear structures and often fragmented. Detection uses:

- PCA axis estimation

- Cluster merging based on colinearity

- Gap-based segment fusion

- Thin bounding box constraints

Vertical Structures (Antenna / Electric Pole)

Detected using:

- Height-to-width ratio filtering

- Vertical alignment constraints

- Minimum height thresholds

Wind Turbine Detection

Detected using:

- Large cluster detection

- Minimum size constraints

- Cluster fusion to prevent fragmentation

Lightweight Neural Segmentation

A small neural model is trained for point-wise classification.

Input features: [x, y, z, reflectivity]

Architecture:

MLP: 4 → 64 → 128 → 128 → 5

Model size: ~25k parameters.

Advantages:

- Fast training

- Low memory usage

- Robust to density variation

The model predicts semantic labels per point, and geometric clustering reconstructs the final objects.

Density Robustness

The system is tested with point densities:

100%, 75%, 50%, 25%.

Robustness is ensured through:

- Density-aware clustering

- Cable segment merging

- Geometry-based filtering

Evaluation Strategy

Since no official bounding boxes are provided, we evaluate using proxy ground truth reconstructed from RGB labels.

Metrics used internally:

- Proxy mAP

- Class-wise detection counts

- Robustness across densities

Results

- Heuristic pipeline is stable across densities

- Cable detection benefits from geometric merging

- Lightweight model helps recover small vertical structures

Why Geometry Works Well Here

Object geometry is highly structured:

Cable → linear

Electric Pole → vertical cylinder

Antenna → small vertical structure

Wind Turbine → large tall structure

By encoding these priors, the detector becomes more stable than purely data-driven approaches.

Final System Advantages

- Robust to point sparsity

- Works without RGB labels

- Very low parameter count

- Fast inference

- Interpretable detections

This hybrid approach provides an effective solution for infrastructure detection in airborne LiDAR data.